PC To Receiver Update

1.Requested Devices

1) Computer: offer new software and procedure for the digital satellite receiver.

2) Digital satellite receiver: The receiver that is going to be upgraded. (Short as “receiver”)

3) Upgrading cable for PC to Unit: A cross RS-232 cable.

2.Operation

1) Turn off STB

2) Open computer and connect the RS-232 cable with STB

3) A. Open update tool

B. Tick “Update Software” in Software column.

C. Click ”Open” to browse and choose flash *.bin file,

(See Picture A)

A

4) If necessary, you will update the Ucode and Logo together with Software, the step is the same as above.

- Ucode:   To protect software stability, normally no need to update, please follow your distributor / supplier’s instruction.

- Logo:    When there is any change for the background picture, start up picture and so on you should update it.

(See Picture B)

B

3. Turn on STB.

The update will begin automatically, and you can see the progress:

(See Picture C)

C

4. Update finished:

1) When the progress is finished (See Picture D), the receiver will reboot itself, and LED of receiver display show numbers, if the receiver doesn’t have LED display, please wait about 1min. the update is finished.

2) Turn off the STB, and remove the RS-232.

3) Turn on STB, and you have now the new version of software, but please make STB Factory Default before start to operate and use, otherwise, STB may not operate properly.

If you want to update again, please click the Restart button, then repeat the process from one to three.

D